# Computer Networks Laboratory

# IT 3095

# Lab instructions

# On

# DESIGN, CONFIGURATION AND SIMULATION OF A SIMPLE WIRED NETWORK USING CISCO PACKET TRACER

# Aim: To design, configuration and simulation of a simple wired network using cisco packet tracer.

# Software Required: CISCO Packet Tracer

# Network Specifications: Two Virtual Local Area Networks (VLANs), one VLAN for Web server.

# Network Design: The network scenario is as shown below:

Figure: Network scenario.

Network configuration: The table below can be referred for the network configuration.

| VLAN No. | Name | Network Address | IP Configurations (Range of IP Address given to Laptop/PC) | Gateway |
| --- | --- | --- | --- | --- |
| VLAN 10 | Office_1 | 192.168.1.0/24 | 192.168.1.2-192.168.1.4 | 192.168.1.1 Router IP address |
| VLAN 20 | Office_2 | 192.168.2.0/24 | 192.168.2.2-192.168.2.5 | 192.168.2.1 Router IP address |

The network configuration in terms of IP address is shown below:

| Network | Network address | IP address to end devices | Default gateway | Interface |
| --- | --- | --- | --- | --- |
| Office_1 | 192.168.1.0 | 192.168.1.2-192.168.1.4 | 192.168.1.1 (for all end devices) | One interface of router to this network will have IP 192.168.1.1 |
| Office_2 | 192.168.2.0 | 192.168.2.2-192.168.2.5 | 192.168.2.1 (for all end devices) | One interface of router to this network will have IP 192.168.2.1 |
| Internet | 10.0.0.0 | 10.0.0.2 | 10.0.0.1 (for all end devices) | One interface of router to this network will have IP 10.0.0.1 |
| Between Ro and R1 | 192.168.3.0 | 192.168.3.1-192.168.3.2 | NA (as it’s not an end device) | An interface of router Ro to this network will have IP 192.168.3.1. and an interface of router R1 to this network will have IP 192.168.3.2 |
| Between R0 and R2 | 192.168.4.0 | 192.168.4.1-192.168.4.2 | NA (as it’s not an end device) | An interface of router Ro to this network will have IP 192.168.4.1. and an interface of router R2 to this network will have IP 192.168.4.2 |
| Between R1 and R2 | 192.168.5.0 | 192.168.5.1-192.168.5.2 | NA (as it’s not an end device) | An interface of router R1 to this network will have IP 192.168.5.1. and an interface of router R2 to this network will have IP 192.168.5.2 |

Default subnet mask for all networks are: 255.255.255.0

### Server Configuration

### The server is located on Internet with Network Address: 10.0.0.0/8

Type of server: HTTP(s) Server hosted on a Server.

IP Address of Server machine 10.0.0.2 (e.g., www.kiit.ac.in)

Default Gateway: 10.0.0.1

Server network is Class-A Networks default with subnet mask: 255.0.0.0

### Cable Specifications:

  - (Between PC/Laptop/Server to Switch and Switch to Routers)

    - Connections   Copper Straight-Through

  - (Between Routers to Routers)

    - Connections  Copper Cross –Over

### Switch Specifications:

  - Type: CISCO 2950T-24 (Switch with VLAN support)

  - IEEE 802.3 Fast Ethernet (FE-Copper)

  - Standard: 100-Base_TX

  - Configure VLANs

### Router Specifications

  - Type: Generic (Router-PT-Empty)

  - Add Hardware interfaces: Fast Ethernet 100 Mbps

  - Network Interface Card (NIC): Network Adaptor : IEEE 802.3 Fast Ethernet (FE-Copper)

  - Standard: 100-Base_TX

  - 4 NICs for Router 1 (R1) & Router 2 (R2)

  - 3 NICs for Router 3 (R3)

  - Routing Protocol: Routing Information Protocol v.1 (RIP v1)

### Intermediate Network specifications:

  - In between R1 – R2 (Network : 192.168.5.0/24)

  - In between R1 – R3 (Network : 192.168.6.0/24)

  - In between R2 – R3 (Network : 192.168.7.0/24)

### PC/laptop/ Server Specifications:

  - End Devices  Generic PC/laptop/Server

### Procedure:

- Basic Configuration:

- Start the Cisco Packet Tracer Software and OPEN it GUI using the icon.

- Place all the components as shown in the network scenario.

- Group the networks, Internet, etc. using rectangle, ellipse, etc. shapes for proper visualization.

- Switch configuration:

VLAN Configuration: (CLI Mode)

  - Click on the switch which you want to configure. For e.g., Switch 0.

  - Click on CLI.

RETURN to get started.  Press Enter

Switch>en

Switch#config t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#vlan 10                                                        as per the requirement

Switch(config-vlan)#name Office_1                                    as per the requirement

Switch(config-vlan)#exit

Switch(config)#exit

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Press Enter

Switch#config t

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#interface range fa0/1-4                                  as per the requirement

Switch(config-if-range)#switchport access vlan 10                as per the requirement

Switch(config-if-range)#exit

Switch(config)#exit

Switch#

%SYS-5-CONFIG_I: Configured from console by console

Press Enter

Switch#

Note: As per above command do configuration for all switches

- Router configuration:

- Double-Click  on  Router  to  open menu. Switch off router first. Then add PT-ROUTER-NM-1CFE NIC card to it by dragging.

- Add such NIC card as per the network scenario. For e.g., for Router_0, we need 4 such NIC card.

- Refer the diagram shown below.

- Connect the all end devices and networking components using appropriate cables.

  - (Between PC/Laptop/Server to Switch and Switch to Routers)

    - Connections   Copper Straight-Through

  - (Between Routers to Routers)

    - Connections  Copper Cross –Over

- IP configuration of routers:

- Give the suitable IP address to all routers interfaces. Double click on router config select the suitable interface  Give the IP address.

- Click on config, and

- Select corresponding NIC

- Give IP address (the second IP address)

- This IP will be used as default gateway for all devices on this network.

- 1st IP is not given to any device. It is used to just recognized the network. It used in Routing configuration.

- Ensure port status is on

n

- Repeat above steps (i.e., IP configuration for all routers)

Configure routing protocols in all routers

- Click on router which you want to configure.

- Select RIP as routing protocols.

- Add all network manually one by one.

Now configure each end devices (PC/Laptop, etc)

- To provide IP address to end device

  - Select the end device

  - Select desktop IP configuration

  - And give the IP address very carefully. Default gateway will be the IP address given to the router interface connected to this VLAN network.

Observation:

- Observe the connectivity among different end devices.

- Observe the HTTP file hosted at the web server using web browser.

- Add simple PDU and observe the packets flow in simulation mode.

- Add complex PDU and observe the packets flow in simulation mode.

Conclusion: Write the conclusion in your won words.

Note: The following screen shorts you need to include in the lab record.